MOTOROLA 6802 EMULATOR

on ESP32

Complete User Manual

Sample programs — Beginner · Intermediate · Advanced

Version 3.0  ·  ESP32 Arduino IDE

Copyright (c) 2026 Kamel Khelif

Open-source software — MIT License

What's new in v3.0

★  NVS Flash slot save/load (8 slots, persistent across resets)

★  S19 Motorola SREC export (xs) and import (xsr)

★  Disassembler expanded to full 6802 instruction set

★  Program B2 (HELLO) corrected — proper null-terminated loop

# Chapter 1 — Introduction and Installation

## 1.1  What is this emulator?

This project emulates the Motorola 6802 8-bit microprocessor on an ESP32 microcontroller. The ESP32 runs a C++ program that simulates every 6802 instruction one by one, exactly as if the original chip were present.

This allows you to learn 8-bit machine language on a classic processor without needing the original 1970s hardware.

ℹ  The 6802 is an 8-bit processor released by Motorola in 1977. Widely used in industry and education, its simplicity makes it an excellent starting point for understanding how a computer works internally.

## 1.2  What you can do

- Write machine-language programs directly from a serial terminal

- Run programs instruction by instruction or at full speed

- Observe registers, memory and flags in real time

- Trigger hardware interrupts (IRQ / NMI) manually

- Save and restore programs to ESP32 Flash (8 slots, persistent)

- Export and import programs in standard Motorola S19 format

- Connect your own GPIO peripherals to the 6802 memory bus

## 1.3  Required hardware and software

| Item | Details | Required |
| --- | --- | --- |
| ESP32 dev board | Any model with USB connector | Yes |
| Arduino IDE | Version 1.8.x or 2.x | Yes |
| ESP32 Arduino core | Install via Board Manager + Preferences library included | Yes |
| Serial terminal (PuTTY) | 115200 baud — Local echo: Force OFF | Yes |
| USB cable | To flash the ESP32 | Yes |

## 1.4  Installation

### Step 1 — ESP32 Arduino core

File → Preferences → Additional Board Manager URLs:

https://raw.githubusercontent.com/espressif/arduino-esp32/gh-pages/package_esp32_index.json

Then: Tools → Board → Board Manager → search ESP32 → Install.

### Step 2 — Open the project

esp32_6802/

├── esp32_6802.ino   ← open this file

├── cpu6802.h

└── cpu6802.cpp

### Step 3 — Compile and upload

Tools → Board → ESP32 Dev Module, select COM port, click Upload.

### Step 4 — Configure PuTTY

| Setting | Value |
| --- | --- |
| Speed (Baud) | 115200 |
| Data bits | 8 |
| Stop bits | 1 |
| Parity | None |
| Local echo | Force OFF ← CRITICAL |
| Local line editing | Force OFF |

⚠  Local echo MUST be 'Force OFF'. If enabled, every character appears twice and commands will not work.

## 1.5  First boot

PuTTY session

========================================

Motorola 6802 Emulator on ESP32  v7

Copyright (c) 2026 Kamel Khelif

MIT License — open source

========================================

Memory is empty. Commands:

w ADDR     write program

load N     restore from Flash slot

xsr        import S19 file

g ADDR     run

q          full help

>

# Chapter 2 — Motorola 6802 Architecture

## 2.1  The registers

┌──────────────────────────────────────────────────┐

│  A    8 bits   Accumulator A  (primary calc reg) │

│  B    8 bits   Accumulator B  (secondary)        │

│  X   16 bits   Index register (array pointer)    │

│  SP  16 bits   Stack Pointer  (stack top)        │

│  PC  16 bits   Program Counter (next instr addr) │

│  CC   8 bits   Condition Codes (flags)           │

└──────────────────────────────────────────────────┘

## 2.2  The CC flags

| Flag | Bit | Meaning |
| --- | --- | --- |
| C | 0 | Carry — unsigned addition overflow |
| V | 1 | Overflow — signed arithmetic overflow |
| Z | 2 | Zero — result is exactly 0 |
| N | 3 | Negative — bit 7 of result is 1 |
| I | 4 | IRQ Mask — 1 = IRQ disabled |
| H | 5 | Half-carry — bit 3→4 carry (BCD) |

## 2.3  Memory map

$0000 ─ $007F   Internal RAM  (128 bytes — zero page)

$0080 ─ $7FFF   Free RAM  (programs and data)

$8000            UART DATA   write=TX  read=RX

$8001            UART STATUS bit0=RX ready  bit1=TX ready

$8010            GPIO_DATA   user peripheral

$8011            GPIO_DIR    user peripheral direction

$F000 ─ $FFFF   High memory (RAM, writable)

$FFF8─$FFF9    IRQ  vector

$FFFA─$FFFB    SWI  vector

$FFFC─$FFFD    NMI  vector

$FFFE─$FFFF    RESET vector

## 2.4  Addressing modes

| Mode | Syntax | Size | Description |
| --- | --- | --- | --- |
| Inherent | CLRA | 1 | Operand implicit in register |
| Immediate | LDAA #$42 | 2 | Constant in instruction |
| Direct | LDAA $10 | 2 | 8-bit address — zero page |
| Extended | LDAA $F100 | 3 | Full 16-bit address |
| Indexed | LDAA 5,X | 2 | Effective addr = X + offset |
| Relative | BEQ label | 2 | Signed offset ±127 — branches only |

# Chapter 3 — Monitor Command Reference

## 3.1  Full command table  (v7)

| Cmd | Full syntax | Function |
| --- | --- | --- |
| r | r | Reset CPU |
| g | g ADDR or g | Go — run from ADDR (or from last saved address) |
| p | p | Pause execution |
| s | s or s N | Step 1 or N instructions with trace |
| d | d | Toggle debug trace ON/OFF |
| x | x | Show all registers |
| v | v [ADDR [N]] | Hex + disassembly view |
| m | m ADDR | Hex memory dump (64 bytes) |
| w | w ADDR or w ADDR VAL | Write mode or single byte write |
| i | i | Trigger IRQ |
| n | n | Trigger NMI |
| slots | slots | List all 8 Flash slots ★NEW |
| save | save N START END [NAME] | Save RAM range to Flash slot ★NEW |
| load | load N | Restore Flash slot to RAM ★NEW |
| del | del N | Erase a Flash slot ★NEW |
| xs | xs ADDR LEN [NAME] | Export S19 to terminal ★NEW |
| xsr | xsr | Import S19 from terminal (paste) ★NEW |
| q | q | Show help |

## 3.2  Write mode

PuTTY session

> w 0100

[WRITE MODE] Enter one hex byte per line.

Type '.' to exit.

$0100: 8E    $0101: 00    $0102: FF

$0103: .

[WRITE MODE END]

>

## 3.3  Verify with v

PuTTY session

> v 0100

Adr    | Octets      | Mnemonique          | Cy

--------+-------------+---------------------+----

>$0100  | 8E 00 FF    | LDS  #$00FF         | 4

$0103  | CE 02 00    | LDX  #$0200         | 4

$0106  | A6 00       | LDAA 0,X            | 3

$0108  | 81 00       | CMPA #$00           | 3

$010A  | 27 04       | BEQ  $010F          | 3

$010C  | B7 80 00    | STAA $8000          | 4

$010F  | 08          | INX                 | 2

$0110  | 20 F4       | BRA  $0106          | 3

$0112  | 20 FE       | BRA  $0112          | 3

ℹ  After the disassembler fix in v7, all 6802 opcodes including LDAA 0,X (indexed mode) are displayed correctly. No more ??? for valid instructions.

# Chapter 4 — Essential Instruction Set

## 4.1  Load and store

| Instruction | Operation | Opcode (hex) |
| --- | --- | --- |
| LDAA #n | A = n | 86 nn |
| LDAA addr | A = mem[addr] | 96 aa (direct) / B6 hh ll (extended) |
| LDAA n,X | A = mem[X+n] | A6 nn (indexed) ← used in B2 |
| STAA addr | mem[addr] = A | 97 aa / B7 hh ll / A7 nn (indexed) |
| LDAB #n | B = n | C6 nn |
| STAB addr | mem[addr] = B | D7 aa / F7 hh ll |
| LDX #nn | X = nn | CE hh ll |
| LDS #nn | SP = nn | 8E hh ll |
| CLRA | A = 0 | 4F |
| CLRB | B = 0 | 5F |
| TAB | B = A | 16 |
| TBA | A = B | 17 |

## 4.2  Arithmetic

| Instruction | Operation | Opcode | Flags |
| --- | --- | --- | --- |
| ADDA #n | A=A+n | 8B nn | H N Z V C |
| ADDB #n | B=B+n | CB nn | H N Z V C |
| SUBA #n | A=A-n | 80 nn | N Z V C |
| INCA | A++ | 4C | N Z V |
| DECA | A-- | 4A | N Z V |
| INCB | B++ | 5C | N Z V |
| DECB | B-- | 5A | N Z V |
| INX | X++ | 08 | Z |
| DEX | X-- | 09 | Z |
| ABA | A=A+B | 1B | H N Z V C |

## 4.3  Branches

| Instruction | Condition | Opcode |
| --- | --- | --- |
| BRA offset | Always | 20 nn |
| BEQ offset | Z=1 | 27 nn |
| BNE offset | Z=0 | 26 nn |
| BMI offset | N=1 | 2B nn |
| BPL offset | N=0 | 2A nn |
| JMP addr | Always (3 bytes) | 7E hh ll |
| JSR addr | Subroutine call | BD hh ll |
| RTS | Return | 39 |

## 4.4  Interrupts and stack

| Instruction | Opcode | Description |
| --- | --- | --- |
| CLI | 0E | Enable IRQ (I=0) |
| SEI | 0F | Disable IRQ (I=1) |
| WAI | 3E | Wait for interrupt — freezes CPU |
| SWI | 3F | Software interrupt |
| RTI | 3B | Return from interrupt (pops 7 bytes) |
| PSHA/PULA | 36/32 | Push/Pull A |
| PSHB/PULB | 37/33 | Push/Pull B |
| NOP | 01 | No operation |

## 4.5  Branch offset formula

offset = target_address − (branch_instruction_address + 2)

Forward:  BEQ at $010A, target $010F

offset = $010F − ($010A+2) = $010F − $010C = +3 = 0x03  ✓

Backward: BRA at $0110, target $0106

offset = $0106 − ($0110+2) = $0106 − $0112 = −12 = 0xF4  ✓

Valid range: −128 (0x80) to +127 (0x7F)

# Chapter 5 — Beginner Programs

These programs use a minimal number of instructions. Prerequisites: know how to use w, v, g and x.

★☆☆  LEVEL: BEGINNER

## Program B1 — Display a single character

Objective: load a value into A and send it to the UART.

6802 Assembly

$0100    8E 00 FF   LDS  #$00FF      Init stack pointer

$0103    86 41      LDAA #$41        A = $41 = ASCII 'A'

$0105    B7 80 00   STAA $8000       Write A to UART → displays 'A'

$0108    20 FE      BRA  $0108       Halt (infinite loop on itself)

PuTTY session

> w 0100

$0100: 8E  $0101: 00  $0102: FF

$0103: 86  $0104: 41

$0105: B7  $0106: 80  $0107: 00

$0108: 20  $0109: FE

.

> g 0100

Terminal output

A

★☆☆  LEVEL: BEGINNER

🔧 FIX  Program B2 corrected in v7 — the loop now terminates correctly on the null byte, and the program ends with BRA $0112 (loop on itself) instead of WAI to avoid garbage output.

## Program B2 — Display the word HELLO

Objective: send several characters using X as a pointer into a null-terminated string.

Key fix: the program ends with BRA $0112 (opcode 20 FE — loop on itself) instead of WAI. This prevents the CPU from executing garbage bytes after halting.

### How the loop works

X → start of string ($0200)

LOOP:

LDAA 0,X    load character at X

CMPA #0     null terminator?

BEQ  done   yes → exit

STAA $8000  no  → send to UART

INX         advance X++

BRA  LOOP   repeat

DONE:

BRA  DONE   halt (loop on itself)

### Offset verification

BEQ at $010A, target $010F:

offset = $010F − ($010A+2) = $010F − $010C = +3 → 0x04

Wait — INX is at $010F, BRA at $0110:

Correct layout:

$010A  27 04   BEQ  $0110   (offset = $0110−$010C = +4)

$010C  B7 80 00 STAA $8000

$010F  08       INX

$0110  20 F4    BRA  $0106   (offset = $0106−$0112 = −12 = 0xF4)

$0112  20 FE    BRA  $0112   (halt — offset = −2 = 0xFE)

### Commented assembly listing

6802 Assembly

Address  Bytes       Mnemonic        Comment

──────── ──────────  ──────────────  ──────────────────────────────

$0100    8E 00 FF    LDS  #$00FF     Init stack pointer

$0103    CE 02 00    LDX  #$0200     X → string data at $0200

$0106    A6 00       LDAA 0,X        Load char at X into A

$0108    81 00       CMPA #0         Null terminator?

$010A    27 04       BEQ  $0110      Yes → jump to BRA $0112 (done)

$010C    B7 80 00    STAA $8000      No  → send char to UART

$010F    08          INX             X++

$0110    20 F4       BRA  $0106      Loop back (0xF4 = −12)

$0112    20 FE       BRA  $0112      Halt — loop on itself

⚠  The BEQ at $010A has offset 0x04 (not 0x03!) because BEQ jumps PAST INX to reach the final BRA. Verify carefully with 'v 0100' before running.

### w commands — program

PuTTY session

> w 0100

$0100: 8E  $0101: 00  $0102: FF

$0103: CE  $0104: 02  $0105: 00

$0106: A6  $0107: 00

$0108: 81  $0109: 00

$010A: 27  $010B: 04

$010C: B7  $010D: 80  $010E: 00

$010F: 08

$0110: 20  $0111: F4

$0112: 20  $0113: FE

.

### w commands — string data at $0200

PuTTY session

> w 0200

$0200: 48   (H)

$0201: 45   (E)

$0202: 4C   (L)

$0203: 4C   (L)

$0204: 4F   (O)

$0205: 0D   (CR)

$0206: 0A   (LF)

$0207: 00   (null terminator — stops the loop)

.

> v 0100

> g 0100

### Expected output

Terminal output

HELLO

✔  HELLO appears exactly once, then the CPU loops on BRA $0112 without sending any more bytes to the UART. No garbage on the terminal.

# Chapter 6 — Intermediate Programs

These programs introduce counting loops, software delays and subroutines.

★★☆  LEVEL: INTERMEDIATE

## Program I1 — Counter 0 to 9

6802 Assembly

$0100    8E 00 FF   LDS  #$00FF

$0103    4F         CLRA             A = 0 (counter)

; loop @ $0104

$0104    8B 30      ADDA #$30        Convert to ASCII

$0106    B7 80 00   STAA $8000       Send digit

$0109    80 30      SUBA #$30        Restore numeric value

$010B    C6 0D      LDAB #$0D

$010D    F7 80 00   STAB $8000       CR

$0110    C6 0A      LDAB #$0A

$0112    F7 80 00   STAB $8000       LF

$0115    4C         INCA

$0116    81 0A      CMPA #10

$0118    27 03      BEQ  $011D       done (offset=+3)

$011A    20 E8      BRA  $0104       loop (offset=0xE8=-24)

$011C    3E         WAI

$011D    20 FE      BRA  $011D

PuTTY session

> w 0100

$0100: 8E  $0101: 00  $0102: FF  $0103: 4F

$0104: 8B  $0105: 30

$0106: B7  $0107: 80  $0108: 00

$0109: 80  $010A: 30

$010B: C6  $010C: 0D

$010D: F7  $010E: 80  $010F: 00

$0110: C6  $0111: 0A

$0112: F7  $0113: 80  $0114: 00

$0115: 4C  $0116: 81  $0117: 0A

$0118: 27  $0119: 03

$011A: 20  $011B: E8

$011C: 3E  $011D: 20  $011E: FD

.

> g 0100

Terminal output

0

1

2

3

4

5

6

7

8

9

★★☆  LEVEL: INTERMEDIATE

## Program I2 — Subroutine: add two numbers

6802 Assembly

; Main @ $0100

$0100    8E 00 FF   LDS  #$00FF

$0103    86 0F      LDAA #15

$0105    C6 0A      LDAB #10

$0107    BD 02 00   JSR  $0200       Call addAB()

$010A    B7 80 00   STAA $8000       Send result ($19=25)

$010D    20 FE      BRA  $010D

; Subroutine addAB @ $0200

$0200    1B         ABA              A = A + B

$0201    39         RTS

⚠  Always end a subroutine with RTS (39). Forgetting it causes a crash.

★★☆  LEVEL: INTERMEDIATE

## Program I3 — Software delay and UART blinking

6802 Assembly

; Main @ $0100

$0100    8E 00 FF   LDS #$00FF

$0103    86 2A      LDAA #$2A   '*'

$0105    B7 80 00   STAA $8000

$0108    BD 02 00   JSR  $0200  delay

$010B    86 2E      LDAA #$2E   '.'

$010D    B7 80 00   STAA $8000

$0110    BD 02 00   JSR  $0200  delay

$0113    20 EE      BRA  $0103  (0xEE=-18)

; Delay @ $0200

$0200    86 64      LDAA #100

$0202    C6 C8      LDAB #200

$0204    5A         DECB

$0205    26 FD      BNE  $0204  inner (0xFD=-3)

$0207    4A         DECA

$0208    26 F8      BNE  $0202  outer (0xF8=-8)

$020A    39         RTS

# Chapter 7 — Advanced Programs

These programs implement interrupts, array algorithms and UART communication.

★★★  LEVEL: ADVANCED

## Program A1 — Complete IRQ handling

Main program loops displaying '.'. IRQ handler fires on 'i' command and prints [IRQ!].

6802 Assembly

; Main @ $0100

$0100    8E 00 FF   LDS #$00FF

$0103    0E         CLI           Enable IRQ

$0104    86 2E      LDAA #'.'

$0106    B7 80 00   STAA $8000

$0109    20 F9      BRA  $0104

; Handler @ $0200  (ends with RTI — NOT RTS)

...send [IRQ!] string...

$0232    3B         RTI

; IRQ vector → $0200

> w FFF8 02   > w FFF9 00

⚠  RTI (3B) is mandatory at the end of every interrupt handler. Using RTS (39) corrupts all registers.

★★★  LEVEL: ADVANCED

## Program A2 — Array sum

Sum a 5-element array. Data at $0010-$0014 (values 10,20,30,40,50). Expected sum: 150 = $96.

6802 Assembly

$0100    8E 00 FF   LDS #$00FF

$0103    CE 00 10   LDX #$0010

$0106    4F         CLRA

$0107    C6 05      LDAB #5

$0109    AB 00      ADDA 0,X

$010B    08         INX

$010C    5A         DECB

$010D    26 FA      BNE  $0109   (0xFA=-6)

$010F    97 15      STAA $15

$0111    B7 80 00   STAA $8000

$0114    20 FE      BRA  $0114

★★★  LEVEL: ADVANCED

## Program A3 — UART echo with counter

Read every character, echo it back, display count every 10 characters.

6802 Assembly

wait: B6 80 01   LDAA $8001   Read STATUS

84 01      ANDA #1      Test RX bit

27 F9      BEQ  wait    Not ready

B6 80 00   LDAA $8000   Read char

B7 80 00   STAA $8000   Echo

...increment counter, display every 10...

# Chapter 8 — Saving and Restoring Programs (Flash Slots)

★  NEW IN v7

The ESP32 has 4 MB of Flash memory. Version 7 uses the NVS (Non-Volatile Storage) partition to save up to 8 program slots. Slots survive power cuts and hardware resets — your programs are always available.

## 8.1  NVS storage format

Each slot stores 4 keys in the NVS namespace '6802':

| NVS Key | Type | Content |
| --- | --- | --- |
| sN_nm | String | Program name (max 31 chars) |
| sN_sa | UInt16 | Start address of the saved range |
| sN_ea | UInt16 | End address of the saved range (inclusive) |
| sN_dt | Bytes | Raw memory bytes from start to end |

N is the slot number (0 to 7). Only the bytes between start and end are stored — you choose the range to keep slot size small.

ℹ  Maximum slot size depends on available NVS space (~20 KB shared across all keys). For typical programs of a few hundred bytes, 8 slots is more than sufficient.

## 8.2  slots — list all slots

PuTTY session

> slots

╔═══╦══════════════════╦═══════════╦═════════╗

║ # ║ Name             ║ Range     ║ Bytes   ║

╠═══╬══════════════════╬═══════════╬═════════╣

║ 0 ║ HELLO            ║ $0100-$0113║     20  ║

║ 1 ║ COUNTER          ║ $0100-$011E║     31  ║

║ 2 ║ (empty)          ║ ---       ║ ---     ║

║ 3 ║ IRQ_DEMO         ║ $0100-$0232║    307  ║

║ 4 ║ (empty)          ║ ---       ║ ---     ║

║ 5 ║ (empty)          ║ ---       ║ ---     ║

║ 6 ║ (empty)          ║ ---       ║ ---     ║

║ 7 ║ (empty)          ║ ---       ║ ---     ║

╚═══╩══════════════════╩═══════════╩═════════╝

## 8.3  save — save a range to a slot

Syntax:  save SLOT START END [NAME]

Saves all bytes from START to END (inclusive) into slot SLOT. NAME is optional (default: 'program').

PuTTY session

; Save the HELLO program (code $0100-$0113 + data $0200-$0207)

; We save the wider range $0100-$0207 to include both:

> save 0 0100 0207 HELLO

[SAVE] Slot 0 saved: "HELLO"  $0100-$0207  (264 bytes)

; Save only the code section:

> save 1 0100 0113 HELLO_CODE

[SAVE] Slot 1 saved: "HELLO_CODE"  $0100-$0113  (20 bytes)

; Save IRQ demo with handler and vectors:

> save 2 0100 0232 IRQ_DEMO

[SAVE] Slot 2 saved: "IRQ_DEMO"  $0100-$0232  (307 bytes)

✔  Include the interrupt vectors in your save range if your program uses IRQ/NMI. Example: save 2 0100 FFFF IRQ_PROG saves everything including $FFF8-$FFFF.

## 8.4  load — restore a slot

Syntax:  load SLOT

Writes all saved bytes back to their original addresses. Also sets go_addr to the saved start address so you can type 'g' immediately.

PuTTY session

> load 0

[LOAD] Slot 0 restored: "HELLO"  $0100-$0207  (264 bytes)  go_addr=$0100

— type 'g' to run

> g

[GO] PC=$0100  type 'p' to pause.

HELLO

⚠  Loading a slot overwrites memory at the saved addresses. Any program already at those addresses will be replaced.

## 8.5  del — erase a slot

PuTTY session

> del 1

[DEL] Slot 1 erased

> slots

║ 1 ║ (empty)  ...

## 8.6  Typical workflow

PuTTY session

1. Write your program:

> w 0100  ... (enter bytes) ...

> w 0200  ... (enter data)  ...

2. Verify:

> v 0100

3. Test:

> g 0100

4. Save to slot:

> save 0 0100 020F MYPROG

5. Next session — just restore and run:

> load 0

> g

# Chapter 9 — S19 Motorola SREC Format

★  NEW IN v7

S19 (also called SREC or Motorola S-Record) is the standard file format that Motorola assemblers and linkers produced for the 6800/6802 family. It is still widely used today in embedded development. Version 7 adds full S19 export and import.

## 9.1  S19 record structure

A S19 file is a plain text file. Each line is one record. Three record types are used:

| Type | Purpose | Address size |
| --- | --- | --- |
| S0 | Header — optional program name | 0000 (always) |
| S1 | Data — bytes to load at a 16-bit address | 16-bit (2 bytes) |
| S9 | End — execution start address | 16-bit (2 bytes) |

### Record layout

S1  13  0100  8E 00 FF CE 02 00 A6 00 81 00 27 04 B7 80 00 08  XX

│   │   │     └─────────────── data bytes ──────────────────┘  │

│   │   └─ address (16-bit big-endian)                          │

│   └─ byte count = addr(2) + data(N) + checksum(1)            │

└─ record type                                        checksum ─┘

Checksum = one's complement of sum of (bytecount + addr_hi + addr_lo + all_data)

### Example S19 file — HELLO program

S19 file

S0 07 0000 48454C4C4F 00 B0       ; header: name='HELLO'

S1 17 0100 8E00FFCE020 0A600810027 04B7800008 20F420FE D2  ; code at $0100

S1 0B 0200 48454C4C4F0D0A00 8C    ; data at $0200: HELLO\r\n\0

S9 03 0100 FB                     ; start address = $0100

ℹ  The checksum is computed on byte_count + address bytes + data bytes, then bitwise inverted (one's complement). The emulator verifies the checksum on every S1 record during import.

## 9.2  xs — export S19 to terminal

Syntax:  xs ADDR LEN [NAME]

Exports LEN bytes starting at ADDR as a complete S19 file to the terminal. Copy the output and save it as a .s19 file on your PC.

PuTTY session

> xs 0100 14 HELLO

; --- Motorola S19 Export ---

; Copyright (c) 2026 Kamel Khelif

S0070000 48454C4C4F 00 B0

S1130100 8E00FFCE0200A60081002704B780000820F420FE XX

S9030100FB

; --- End of S19 ---

✔  To save to a file: in PuTTY, use Logging (Session → Logging) to capture the terminal output to a .txt file, then rename it .s19.

## 9.3  xsr — import S19 from terminal

Syntax:  xsr

Switches the monitor into S19 receive mode. Paste your S19 file into PuTTY line by line. The emulator processes each record, verifies the checksum and writes the data to RAM. Type '.' on a blank line to exit import mode.

PuTTY session

> xsr

[XSR] S19 import mode.

Paste S19 records line by line.

Type '.' to exit import mode.

S0070000 48454C4C4F 00 B0

[S19] Header record received

S1130100 8E00FFCE0200...

[S19] $0100 +16 bytes OK

S1 0B 0200 48454C4C4F0D0A00 8C

[S19] $0200 +8 bytes OK

S9030100FB

[S19] End record — start address: $0100

[S19] Import complete. Type 'g' to run.

>

> g

HELLO

⚠  If a checksum error is reported ([S19] ERROR: checksum mismatch), the record was corrupted during paste. Re-paste that line and continue.

## 9.4  Using S19 with a PC assembler

You can write 6802 assembly on your PC using any Motorola 6800/6802 assembler and produce a .s19 file. Common free tools:

| Tool | Platform | Notes |
| --- | --- | --- |
| ASM6800 (various) | Windows / Linux | Classic Motorola syntax |
| as6800 (ASXXXX) | Cross-platform | Part of the ASXXXX package |
| vasm | Windows / Linux / macOS | Supports 6800 target |
| Any 6800 assembler | — | Must produce S19/SREC output |

Typical assembler → emulator workflow:

Workflow

1. Write source: prog.asm

2. Assemble:     as6800 -l prog.asm -o prog.s19

3. In PuTTY:     > xsr

4. Paste prog.s19 contents

5. Type '.'

6. Run:          > g

# Chapter 10 — Debugging Guide

## 10.1  General workflow

- Enter code with w ADDR

- Verify with v ADDR — check every byte and every mnemonic

- Check vectors with m FFF8 if using interrupts

- Type r then g ADDR

- If something goes wrong: p then x

## 10.2  Branch offset formula

offset = target − (branch_address + 2)    valid: −128 to +127

Out of range → use JMP addr (7E hh ll, unlimited)

## 10.3  Common problems

| Symptom | Cause | Fix |
| --- | --- | --- |
| CPU jumps to random address | Wrong branch offset | Recalculate: target − (BRx_addr+2) |
| Garbage output on terminal | Missing BRA $x (loop on itself) at end | Replace WAI with 20 FE |
| HELLO prints in an infinite loop | Loop exit condition never met — wrong BEQ offset | Verify BEQ offset with v 0100 |
| IRQ ignored | CLI not executed | Add 0E (CLI) after LDS #$00FF |
| Crash after JSR | RTS missing | Verify last byte is 39 (RTS) |
| Crash after interrupt | RTI missing — RTS used | Replace 39 with 3B (RTI) |
| S19 checksum error | Paste was corrupted | Re-paste the failing line |
| load overwrites wrong addresses | Saved range too wide | Use xs first to check what was saved |
| 'w' does not respond | Local echo ON in PuTTY | Local echo = Force OFF |

## 10.4  Inspecting the stack

After LDS #$00FF : SP = $00FF

After JSR        : SP = $00FD  (2 bytes for return address)

After IRQ        : SP = $00F8  (7 bytes: PC X A B CC)

> m 00F8   ← inspect stack after IRQ

# Appendix A — ASCII table

| Hex | Char | Hex | Char | Hex | Char | Hex | Char |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 20 | (sp) | 30 | 0 | 41 | A | 61 | a |
| 21 | ! | 31 | 1 | 42 | B | 62 | b |
| 22 | " | 32 | 2 | 43 | C | 63 | c |
| 23 | # | 33 | 3 | 44 | D | 64 | d |
| 24 | $ | 34 | 4 | 45 | E | 65 | e |
| 25 | % | 35 | 5 | 46 | F | 66 | f |
| 26 | & | 36 | 6 | 47 | G | 67 | g |
| 27 | ' | 37 | 7 | 48 | H | 68 | h |
| 28 | ( | 38 | 8 | 49 | I | 69 | i |
| 29 | ) | 39 | 9 | 4A | J | 6A | j |
| 2A | * | 3A | : | 4B | K | 6B | k |
| 2B | + | 3B | ; | 4C | L | 6C | l |
| 2C | , | 3C | < | 4D | M | 6D | m |
| 2D | - | 3E | > | 4E | N | 6E | n |
| 2E | . | 3F | ? | 4F | O | 6F | o |
| 2F | / | 40 | @ | 50 | P | 70 | p |
| 0D | CR | 0A | LF | 5B | [ | 5D | ] |

# Appendix B — Essential opcodes

| Opcode | Mnemonic | Description |
| --- | --- | --- |
| 01 | NOP | No operation |
| 0E | CLI | Enable IRQ |
| 0F | SEI | Disable IRQ |
| 08 | INX | X++ |
| 09 | DEX | X-- |
| 16 | TAB | B=A |
| 17 | TBA | A=B |
| 1B | ABA | A=A+B |
| 20 nn | BRA | Branch always |
| 26 nn | BNE | Branch Z=0 |
| 27 nn | BEQ | Branch Z=1 |
| 32 | PULA | Pull A |
| 33 | PULB | Pull B |
| 36 | PSHA | Push A |
| 37 | PSHB | Push B |
| 39 | RTS | Return from subroutine |
| 3B | RTI | Return from interrupt |
| 3E | WAI | Wait for interrupt |
| 3F | SWI | Software interrupt |
| 40 | NEGA | Negate A |
| 4A | DECA | A-- |
| 4C | INCA | A++ |
| 4F | CLRA | A=0 |
| 5A | DECB | B-- |
| 5C | INCB | B++ |
| 5F | CLRB | B=0 |
| 7E hh ll | JMP | Absolute jump |
| 86 nn | LDAA # | A=n |
| 8B nn | ADDA # | A=A+n |
| 8E hh ll | LDS # | SP=nn |
| 97 aa | STAA dir | mem[aa]=A |
| A6 nn | LDAA idx | A=mem[X+n] |
| AB nn | ADDA idx | A=A+mem[X+n] |
| B6 hh ll | LDAA ext | A=mem[addr] |
| B7 hh ll | STAA ext | mem[addr]=A |
| BD hh ll | JSR | Call subroutine |
| C6 nn | LDAB # | B=n |
| CE hh ll | LDX # | X=nn |
| 7C hh ll | INC | mem[addr]++ |

# Appendix C — Glossary

| Term | Definition |
| --- | --- |
| Accumulator | Working register (A or B) — arithmetic and logic happen here |
| Big-endian | MSB stored first (e.g. $0200 stored as $02 $00) |
| Branch offset | Signed byte: offset = target − (branch_addr + 2) |
| CLI / SEI | Clear/Set I flag — enable/disable maskable IRQs |
| Flash / NVS | Non-Volatile Storage on the ESP32 — persists across resets |
| Handler | Subroutine for interrupts — must end with RTI not RTS |
| IRQ | Maskable interrupt — ignored when I flag is set |
| NMI | Non-Maskable Interrupt — always serviced |
| NVS | Non-Volatile Storage — ESP32 key-value store used for slots |
| Opcode | First byte of an instruction (e.g. $86 = LDAA immediate) |
| RTI | Return from interrupt — pops 7 bytes (CC B A X PC) |
| RTS | Return from subroutine — pops 2 bytes (PC) |
| S19 / SREC | Motorola S-Record — standard text format for binary programs |
| Slot | One of 8 NVS save locations for 6802 programs |
| SP | Stack Pointer — decremented on PUSH, incremented on PULL |
| UART | Serial port, memory-mapped at $8000/$8001 |
| Vector | 16-bit address in high memory pointing to an interrupt handler |
| WAI | Wait for Interrupt — freezes CPU until IRQ or NMI |
| Zero page | $0000-$00FF — faster 2-byte addressing |

Copyright (c) 2026 Kamel Khelif  —  MIT License  —  Open-source software