MOTOROLA 6802 EMULATOR

on ESP32

TECHNICAL REFERENCE MANUAL

Internal Architecture · Memory System · Instruction Execution

Interrupt Handling · I/O Dispatch · Monitor · NVS · S19

Troubleshooting Guide

Version 7.0  ·  Arduino IDE / ESP32

Copyright (c) 2026 Kamel Khelif

MIT License — Open-source software

# Chapter 1 — Project Overview and Architecture

## 1.1  Purpose and scope

This document is the complete technical reference for the Motorola 6802 emulator running on an ESP32 microcontroller (Arduino IDE). It covers every internal mechanism of the emulator: the CPU state machine, memory routing, instruction execution pipeline, interrupt subsystem, memory-mapped I/O dispatch, the serial monitor, the NVS Flash storage layer, and the S19 file format processor.

This manual is intended for developers who need to understand, extend or troubleshoot the emulator at source-code level. It is not a tutorial for 6802 assembly language.

## 1.2  Source file structure

esp32_6802/

├── cpu6802.h        ← Public API: CPU6802 struct, constants, prototypes

├── cpu6802.cpp      ← Emulator core: memory, instructions, interrupts,

│                      disassembler, debug helpers

└── esp32_6802.ino   ← Arduino sketch: monitor, NVS slots, S19, loop()

## 1.3  High-level execution model

The emulator is entirely cooperative and single-threaded. Arduino's loop() function runs continuously and interleaves three activities:

┌──────────────────────────────────────────────────────────────────────┐

│                         loop()  (called ~240 Hz at 240 MHz ESP32)    │

│                                                                       │

│  ① while(Serial.available())          Serial monitor input           │

│       process_monitor_command()        Parse and execute commands     │

│                                                                       │

│  ② if(running && !cpu.halted)          CPU execution                 │

│       for i in 0..STEPS_PER_LOOP(200)                                │

│           cpu6802_step()               Execute ONE instruction        │

│           if(cpu.halted) break                                        │

│                                                                       │

│  ③ if(millis()-last_gpio > 50ms)       GPIO polling                  │

│       check PIN_NMI / PIN_IRQ                                        │

│                                                                       │

│  ④ yield()                             ESP32 internal tasks           │

└──────────────────────────────────────────────────────────────────────┘

ℹ  NOTE  STEPS_PER_LOOP = 200 means the emulator executes 200 6802 instructions per loop() call. At ~240 loop calls/sec the effective emulated frequency is ~48,000 instructions/sec — far below the real 1 MHz 6802, but sufficient for educational use.

## 1.4  CPU6802 structure — complete field reference

Defined in cpu6802.h. This is the single object that represents the entire CPU state. One global instance is declared in esp32_6802.ino as:

CPU6802 cpu;

| Field | C type | Size | Description |
| --- | --- | --- | --- |
| A | uint8_t | 8 bit | Accumulator A — primary arithmetic register |
| B | uint8_t | 8 bit | Accumulator B — secondary arithmetic register |
| X | uint16_t | 16 bit | Index register — base for indexed addressing |
| SP | uint16_t | 16 bit | Stack Pointer — points to last occupied stack byte |
| PC | uint16_t | 16 bit | Program Counter — address of next instruction to fetch |
| CC | uint8_t | 8 bit | Condition Code register — H I N Z V C flags (bits 7:6 = 1) |
| ram_internal[] | uint8_t[128] | 128 B | Internal RAM ($0000–$007F), physically separate array |
| memory[] | uint8_t[65536] | 64 KB | Full address-space array — external RAM + ROM area |
| halted | bool | 1 bit | Set by illegal opcode or explicit halt — stops cpu6802_step() |
| irq_pending | bool | 1 bit | Set by cpu6802_irq() — consumed at start of next step |
| nmi_pending | bool | 1 bit | Set by cpu6802_nmi() — higher priority, consumed first |
| cycles | uint32_t | 32 bit | Total clock cycles accumulated since last reset |
| instr_count | uint32_t | 32 bit | Total instructions executed since last reset |
| debug_mode | bool | 1 bit | When true, every instruction is printed to Serial |
| io_read_cb | function ptr | 4 B | Callback: uint8_t f(uint16_t addr) — called for $8000–$8FFF reads |
| io_write_cb | function ptr | 4 B | Callback: void f(uint16_t addr, uint8_t val) — called for writes |

ℹ  NOTE  Total RAM consumed by CPU6802 on the ESP32 heap: 128 + 65536 + registers + state = ~65.8 KB. The ESP32 has 320 KB SRAM so this is easily accommodated.

# Chapter 2 — Memory System

## 2.1  Full memory map

Address       Size    Region              Implementation

─────────     ─────   ──────────────────  ─────────────────────────────────

$0000─$007F   128 B   Internal RAM        cpu->ram_internal[] (separate array)

$0080─$7FFF   ~32 KB  External RAM        cpu->memory[addr]

$8000─$8FFF   4 KB    Memory-mapped I/O   Dispatched to io_read/write_cb()

$9000─$EFFF   24 KB   Free RAM            cpu->memory[addr]

$F000─$FFFF   4 KB    High area / ROM     cpu->memory[addr]  (writable)

$FFF8─$FFF9  2 B    IRQ  vector         READ16($FFF8) on interrupt

$FFFA─$FFFB  2 B    SWI  vector         READ16($FFFA) on SWI

$FFFC─$FFFD  2 B    NMI  vector         READ16($FFFC) on NMI

$FFFE─$FFFF  2 B    RESET vector        READ16($FFFE) on cpu6802_reset()

ℹ  NOTE  In this implementation the 'ROM' area ($F000–$FFFF) is fully writable. The comment in cpu6802_write() reads: '// Allow writes (no protection in this build)'. This is intentional — the serial monitor uses w FFF8 to set interrupt vectors.

## 2.2  Memory routing — cpu6802_read()

Every byte access by the emulated CPU goes through cpu6802_read(). The routing priority is:

cpu6802_read(addr)

│

├─ addr < $0080 (128) ?

│      YES → return cpu->ram_internal[addr]

│

├─ $8000 ≤ addr ≤ $8FFF ?

│      YES → io_read_cb set?

│              YES → return io_read_cb(addr)   ← user callback

│              NO  → built-in UART handler:

│                    addr=$8000 → Serial.read() or 0x00

│                    addr=$8001 → status byte (TX=bit1, RX=bit0)

│                    other      → return 0xFF

│

└─ else → return cpu->memory[addr]             ← normal RAM

## 2.3  Memory routing — cpu6802_write()

cpu6802_write(addr, val)

│

├─ addr < $0080 ?

│      YES → cpu->ram_internal[addr] = val

│

├─ $8000 ≤ addr ≤ $8FFF ?

│      YES → io_write_cb set?

│              YES → io_write_cb(addr, val)   ← user callback; return

│              NO  → addr=$8000 → Serial.write(val)

│                    return  (writes to other I/O addresses are ignored)

│

└─ else → cpu->memory[addr] = val

(includes $F000–$FFFF — ROM area is writable)

## 2.4  Big-endian 16-bit access

The 6802 is a big-endian processor: the high byte is stored at the lower address. cpu6802_read16() implements this:

cpu6802.cpp — cpu6802_read16()

uint16_t cpu6802_read16(CPU6802 *cpu, uint16_t addr) {

return ((uint16_t)cpu6802_read(cpu, addr) << 8)

| cpu6802_read(cpu, addr + 1);

}

FETCH16() in the step function uses this with auto-increment:

#define FETCH16()  (cpu->PC += 2, READ16(cpu->PC - 2))

// Increments PC by 2 THEN reads the word that was at old PC.

## 2.5  Internal vs external RAM duality

The 6802 chip physically contains 128 bytes of RAM at $0000–$007F. In this emulator they are stored in a SEPARATE array (ram_internal[128]) rather than at the beginning of memory[65536]. Both routing functions check addr < RAM_INTERNAL first and redirect accordingly.

⚠  WARNING  If you write directly to cpu->memory[0x0000] bypassing the API, you will NOT see the data when the CPU reads from address $0000. Always use cpu6802_read/write() or the w monitor command.

## 2.6  Memory-mapped I/O — adding peripherals

The io_read_cb and io_write_cb function pointers in CPU6802 are set in esp32_6802.ino:

cpu.io_read_cb  = user_io_read;

cpu.io_write_cb = user_io_write;

These callbacks are called for EVERY access to $8000–$8FFF. The default implementation handles UART at $8000/$8001:

| Address | Direction | Bit(s) | Meaning |
| --- | --- | --- | --- |
| $8000 | Write | 7:0 | Byte sent to ESP32 Serial (UART TX) |
| $8000 | Read | 7:0 | Byte received from Serial (UART RX) or $00 if none |
| $8001 | Read | 0 | 1 = at least one byte waiting in Serial RX buffer |
| $8001 | Read | 1 | Always 1 — TX is always ready on ESP32 |
| $8010 | R/W | 7:0 | GPIO_DATA — user-defined, wired in user_io_read/write() |
| $8011 | R/W | 7:0 | GPIO_DIR — user-defined |

# Chapter 3 — Condition Code Register (CC)

## 3.1  Bit layout

Bit:   7    6    5    4    3    2    1    0

│    │    │    │    │    │    │    │

Flag:  1    1    H    I    N    Z    V    C

│    │    │    │    │    │    │    │

│    │    │    │    │    │    │    └─ C  Carry

│    │    │    │    │    │    └────── V  Overflow (signed)

│    │    │    │    │    └─────────── Z  Zero

│    │    │    │    └──────────────── N  Negative

│    │    │    └───────────────────── I  IRQ Mask

│    │    └────────────────────────── H  Half-carry (BCD)

│    └─────────────────────────────── always 1 (hardware)

└──────────────────────────────────── always 1 (hardware)

After cpu6802_init(), CC is initialised to:

cpu->CC = CC_BIT6 | CC_BIT7 | CC_I;

//       = 0b11000000 | 0b00010000

//       = 0xD0  (bits 7,6 always 1; I=1 masks IRQ until CLI is called)

## 3.2  Flag-update macros

| Macro | Expansion | Used for |
| --- | --- | --- |
| SET_FLAG(f) | CC ¦= f | Force a flag to 1 |
| CLR_FLAG(f) | CC &= ~f | Force a flag to 0 |
| TST_FLAG(f) | (CC & f) != 0 | Test a flag — returns bool |
| SET_OR_CLR(f,c) | if(c) SET else CLR | Conditional set/clear |
| UPD_NZ8(val) | N=bit7(val), Z=(val==0) | Update N and Z after 8-bit result |
| UPD_NZ16(val) | N=bit15(val), Z=(val==0) | Update N and Z after 16-bit result |

## 3.3  Flag semantics — detailed

### C — Carry flag

Set when an addition produces a carry out of bit 7, or when a subtraction requires a borrow. Also used as input to ADC/SBC and as the output of shift instructions.

// In add8():

SET_OR_CLR(CC_C, res16 > 0xFF);  // carry if 8-bit result overflows

// In sub8():

SET_OR_CLR(CC_C, res16 > 0xFF);  // borrow if 8-bit result underflows

### V — Overflow flag (signed)

Set when a signed arithmetic operation produces a result outside the range –128 to +127. The test is based on the sign bits of the operands and result:

// In add8():

SET_OR_CLR(CC_V, ((a ^ res) & (b ^ res)) & 0x80);

// Both operands same sign, result different sign = overflow

// In sub8():

SET_OR_CLR(CC_V, ((a ^ b) & (a ^ res)) & 0x80);

// Operands different sign, result sign matches subtrahend = overflow

### Z — Zero flag

Set when the result of an operation is exactly 0. Part of every UPD_NZ8/UPD_NZ16 call. Also set individually by INX/DEX when X reaches 0.

### N — Negative flag

Set when bit 7 of the 8-bit result is 1, or bit 15 of the 16-bit result is 1. Indicates a negative number in two's complement representation.

### I — IRQ Mask

When I=1 (set), maskable IRQ interrupts are ignored. When I=0 (clear), IRQs are serviced. Cleared by CLI (0x0E), set by SEI (0x0F) and automatically set by do_interrupt() to prevent nested IRQs.

### H — Half-carry

Set when there is a carry from bit 3 into bit 4 during addition. Used only by the DAA (Decimal Adjust Accumulator) instruction for BCD arithmetic.

// In add8():

SET_OR_CLR(CC_H, ((a & 0x0F) + (b & 0x0F) + carry) > 0x0F);

## 3.4  Flag update summary by instruction group

| Instruction group | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- |
| LDAA/LDAB/STAA/STAB | – | – | ● | ● | 0 | – |
| LDX/LDS/STX/STS | – | – | ● | ● | 0 | – |
| ADDA/ADDB (add8) | ● | – | ● | ● | ● | ● |
| ADCA/ADCB (add8 with carry) | ● | – | ● | ● | ● | ● |
| SUBA/SUBB (sub8) | – | – | ● | ● | ● | ● |
| SBCA/SBCB (sub8 with borrow) | – | – | ● | ● | ● | ● |
| ANDA/ANDB/ORAA/ORAB/EORA/EORB | – | – | ● | ● | 0 | – |
| BITA/BITB (AND without store) | – | – | ● | ● | 0 | – |
| CMPA/CMPB/CPX (sub without store) | – | – | ● | ● | ● | ● |
| INCA/INCB/INC mem | – | – | ● | ● | ● | – |
| DECA/DECB/DEC mem | – | – | ● | ● | ● | – |
| INX/DEX | – | – | – | ● | – | – |
| ASLA/ASLB/ASL | – | – | ● | ● | ● | ● |
| LSRA/LSRB/LSR | – | – | 0 | ● | ● | ● |
| ROLA/ROLB/ROL | – | – | ● | ● | ● | ● |
| RORA/RORB/ROR | – | – | ● | ● | ● | ● |
| CLI/SEI | – | ● | – | – | – | – |
| TAP (A→CC) | ● | ● | ● | ● | ● | ● |

● = updated by instruction   0 = forced to 0   – = unchanged

# Chapter 4 — Instruction Execution Pipeline

## 4.1  cpu6802_step() — the main execution engine

cpu6802_step() is called 200 times per loop() iteration when the CPU is running. It performs one complete instruction cycle.

cpu6802_step(cpu)

│

├─ cpu->halted? → return 0 immediately

│

├─ nmi_pending?                            ← NMI always wins

│     → do_interrupt(VEC_NMI)  return 12

│

├─ irq_pending AND NOT TST_FLAG(CC_I)?     ← IRQ only if unmasked

│     → do_interrupt(VEC_IRQ)  return 12

│

├─ pc_before = cpu->PC   ← save PC BEFORE fetch (needed by WAI, debug)

│

├─ opcode = FETCH8()     ← reads mem[PC] and increments PC

│

├─ if(debug_mode)        ← print disassembly + register state

│

├─ switch(opcode)        ← 256 cases, 72 instructions implemented

│     decode operands (FETCH8/16, addr_direct/extended/indexed)

│     execute operation

│     update CC flags

│     assign cycles

│

├─ cpu->cycles     += cycles

├─ cpu->instr_count += 1

└─ return cycles

## 4.2  Addressing mode functions

Three functions compute effective addresses from the byte(s) following the opcode:

| Function | Mode | Bytes consumed | Effective address (EA) | Example |
| --- | --- | --- | --- | --- |
| addr_direct() | Direct | 1 | EA = FETCH8() (0–255, zero page) | LDAA $42 → EA=$0042 |
| addr_extended() | Extended | 2 | EA = FETCH16() (0–65535) | LDAA $F100 → EA=$F100 |
| addr_indexed() | Indexed | 1 | EA = (X + FETCH8()) & 0xFFFF | LDAA 5,X → EA=X+5 |

static uint16_t addr_direct(CPU6802 *cpu)   { return FETCH8(); }

static uint16_t addr_extended(CPU6802 *cpu) { return FETCH16(); }

static uint16_t addr_indexed(CPU6802 *cpu)  { return (cpu->X + FETCH8()) & 0xFFFF; }

// Relative (branches): offset is signed, applied to current PC

case 0x27: { int8_t off=(int8_t)FETCH8(); if(TST_FLAG(CC_Z)) cpu->PC+=off; } // BEQ

## 4.3  Stack macros — PUSH/PULL

The 6802 stack grows DOWNWARD. SP points to the last byte WRITTEN. PUSH decrements SP first, PULL increments SP first:

#define PUSH8(v)   do { WRITE(cpu->SP--, (v) & 0xFF); } while(0)

// Write to current SP, then SP--

#define PULL8()    (READ(++cpu->SP))

// SP++ first, then read from new SP

#define PUSH16(v)  do { PUSH8((v) & 0xFF); PUSH8(((v) >> 8) & 0xFF); } while(0)

// Push LSB first, then MSB → MSB ends up at lower address (big-endian on stack)

#define PULL16()   ((uint16_t)(PULL8() << 8) | PULL8())

// Pull MSB first (it was at lower address), then LSB

Example: PUSH16($0123) with SP=$00FF

Step 1: PUSH8($23) → mem[$00FF]=$23, SP=$00FE

Step 2: PUSH8($01) → mem[$00FE]=$01, SP=$00FD

Memory layout after push:

$00FD: (SP points here — empty)

$00FE: $01  ← high byte

$00FF: $23  ← low byte

PULL16() → reads $00FE=$01 (MSB), reads $00FF=$23 (LSB) → $0123, SP=$00FF

## 4.4  Arithmetic helpers — add8() and sub8()

All 8-bit addition and subtraction operations route through these two helpers to guarantee consistent flag updates:

cpu6802.cpp

static uint8_t add8(CPU6802 *cpu, uint8_t a, uint8_t b, uint8_t carry) {

uint16_t res16 = (uint16_t)a + b + carry;

uint8_t  res   = (uint8_t)res16;

SET_OR_CLR(CC_H, ((a&0x0F)+(b&0x0F)+carry) > 0x0F); // H: nibble carry

SET_OR_CLR(CC_C, res16 > 0xFF);                      // C: byte overflow

SET_OR_CLR(CC_V, ((a^res)&(b^res))&0x80);            // V: sign change

UPD_NZ8(res);                                         // N, Z

return res;

}

static uint8_t sub8(CPU6802 *cpu, uint8_t a, uint8_t b, uint8_t borrow) {

uint16_t res16 = (uint16_t)a - b - borrow;

uint8_t  res   = (uint8_t)res16;

SET_OR_CLR(CC_C, res16 > 0xFF);           // C: borrow

SET_OR_CLR(CC_V, ((a^b)&(a^res))&0x80);   // V: sign change

UPD_NZ8(res);

return res;

}

ℹ  NOTE  CMPA and CMPB use sub8() but discard the result — they update flags without modifying the accumulator. BITA/BITB use AND but similarly discard the result.

## 4.5  WAI — Wait for Interrupt

WAI (opcode $3E) is the most subtle instruction in this emulator. The real 6802 freezes the bus and waits. Our emulator cannot block Arduino's loop() — it must remain responsive. The implementation:

cpu6802.cpp — WAI implementation

case 0x3E:  // WAI — Wait for interrupt

PUSH16(cpu->PC);  // Save full context

PUSH16(cpu->X);

PUSH8(cpu->A);

PUSH8(cpu->B);

PUSH8(cpu->CC);

cpu->PC = pc_before;  // ← KEY: PC stays at WAI instruction

cycles = 9;

break;

⚠  WARNING  cpu->PC = pc_before is the critical line. Without it, PC advances past WAI and the next instruction (random bytes) executes. The bug was: WAI pushed context but didn't restore PC, causing the CPU to execute whatever follows in memory. Fixed by capturing pc_before = cpu->PC BEFORE the opcode fetch.

WAI behaviour in this emulator:

Before WAI:  PC=$0112  (points at WAI opcode $3E)

During WAI:  Context pushed to stack, then PC=$0112 again

Each loop(): cpu6802_step() fetches $3E again → pushes again → overwrites stack

On IRQ/NMI:  nmi_pending/irq_pending checked FIRST → do_interrupt() takes over

The stack already has the correct return address

Result: WAI loops harmlessly on itself until an interrupt fires.

## 4.6  Unknown opcode handling

Unrecognised opcodes do NOT halt the CPU. They consume 2 cycles and the emulator prints a warning in debug mode:

default:

if (cpu->debug_mode) {

Serial.print(F("[WARN] Unknown opcode: 0x"));

Serial.print(opcode, HEX);

Serial.print(F(" @ 0x"));

Serial.println(pc_before, HEX);

}

cycles = 2;

break;

ℹ  NOTE  To halt the CPU on unknown opcodes, replace 'cycles = 2;' with 'cpu->halted = true;'. This is more faithful to real hardware but may be inconvenient during development.

# Chapter 5 — Interrupt Subsystem

## 5.1  Interrupt sources

| Source | Type | Maskable? | How triggered | Vector |
| --- | --- | --- | --- | --- |
| IRQ | Hardware maskable | Yes — I flag | cpu6802_irq() or PIN_IRQ GPIO | $FFF8–$FFF9 |
| NMI | Hardware non-maskable | No — always fires | cpu6802_nmi() or PIN_NMI GPIO | $FFFC–$FFFD |
| SWI | Software interrupt | No | Opcode $3F | $FFFA–$FFFB |
| WAI | Wait / pre-save | N/A | Opcode $3E | (any of the above on wake) |

## 5.2  Interrupt priority in cpu6802_step()

At the start of every step, the two pending flags are checked in this order:

// 1. NMI — always serviced first, regardless of I flag

if (cpu->nmi_pending) {

cpu->nmi_pending = false;

do_interrupt(cpu, VEC_NMI);

return 12;

}

// 2. IRQ — only if I flag is clear (not masked)

if (cpu->irq_pending && !TST_FLAG(CC_I)) {

cpu->irq_pending = false;

do_interrupt(cpu, VEC_IRQ);

return 12;

}

ℹ  NOTE  The pending flags are set by the external trigger functions but consumed (cleared) inside cpu6802_step(). This means interrupts are edge-sensitive: calling cpu6802_irq() twice before a step is serviced results in only ONE interrupt being taken.

## 5.3  do_interrupt() — context save and vector fetch

Called for IRQ, NMI and SWI. Saves the full CPU context (7 bytes) on the stack then loads PC from the vector:

static void do_interrupt(CPU6802 *cpu, uint16_t vector) {

PUSH16(cpu->PC);    // 2 bytes — return address

PUSH16(cpu->X);     // 2 bytes — index register

PUSH8(cpu->A);      // 1 byte  — accumulator A

PUSH8(cpu->B);      // 1 byte  — accumulator B

PUSH8(cpu->CC);     // 1 byte  — condition codes

SET_FLAG(CC_I);     // mask further IRQs

cpu->PC = READ16(vector);  // jump to handler

cpu->cycles += 12;

}

Stack layout after do_interrupt() (SP was $00FF before):

Address   Content   Register saved

$00FF     PC_hi     Return address high byte

$00FE     PC_lo     Return address low byte

$00FD     X_hi      Index register high byte

$00FC     X_lo      Index register low byte

$00FB     A         Accumulator A

$00FA     B         Accumulator B

$00F9     CC        Condition Codes  ← SP now points here

SP = $00F8 after do_interrupt() (points to next free slot below CC)

## 5.4  do_rti() — context restore

RTI (opcode $3B) calls do_rti(), which restores the 7 bytes in REVERSE order:

static void do_rti(CPU6802 *cpu) {

cpu->CC = PULL8();   // restore CC first

cpu->B  = PULL8();   // then B

cpu->A  = PULL8();   // then A

cpu->X  = PULL16();  // then X (2 bytes)

cpu->PC = PULL16();  // then PC (2 bytes) — returns to interrupted code

cpu->cycles += 10;

}

⚠  WARNING  RTI (3B) and RTS (39) are NOT interchangeable. RTS only pops 2 bytes (PC). RTI pops 7 bytes. Using RTS in an interrupt handler leaves X, A, B, CC on the stack, corrupts SP, and causes a crash.

## 5.5  GPIO polling — physical interrupt buttons

The sketch checks physical GPIO buttons for IRQ/NMI every 50 ms:

if (millis() - last_gpio_check > 50) {

last_gpio_check = millis();

if (PIN_NMI >= 0 && digitalRead(PIN_NMI) == LOW) {

cpu6802_nmi(&cpu);

delay(200);  // debounce

}

if (PIN_IRQ >= 0 && digitalRead(PIN_IRQ) == LOW) {

cpu6802_irq(&cpu);

delay(200);

}

}

⚠  WARNING  GPIO0 on the ESP32 is the BOOT button. It is pulled LOW at every power-on and reset. Never assign PIN_NMI or PIN_IRQ to GPIO0 — it triggers an immediate NMI loop at startup. Both are set to -1 (disabled) in this build.

## 5.6  Setting interrupt vectors

Interrupt vectors are 16-bit addresses stored big-endian in high memory. Use the w command to set them:

; Route IRQ to handler at $0200

> w FFF8

$FFF8: 02     ← high byte of $0200

$FFF9: 00     ← low byte

; Route NMI to handler at $0300

> w FFFC

$FFFC: 03

$FFFD: 00

# Chapter 6 — Initialisation and Reset Sequence

## 6.1  cpu6802_init() — cold start

void cpu6802_init(CPU6802 *cpu) {

memset(cpu, 0, sizeof(CPU6802));       // Zero all fields, all memory

cpu->CC = CC_BIT6 | CC_BIT7 | CC_I;   // CC = 0xD0: bits 7,6=1; I=1

cpu->SP = 0x00FF;                      // Default stack top

cpu->io_read_cb  = NULL;               // No I/O callbacks yet

cpu->io_write_cb = NULL;

cpu->debug_mode  = false;

}

After init, ALL 64 KB of memory is zero. The RESET vector at $FFFE/$FFFF is $0000. The CPU does NOT auto-start after init — the sketch must either set a RESET vector and call cpu6802_reset(), or directly set cpu.PC and cpu.running.

## 6.2  cpu6802_reset() — warm reset

void cpu6802_reset(CPU6802 *cpu) {

cpu->CC  |= CC_I;         // Mask IRQ

cpu->halted      = false;

cpu->irq_pending = false;

cpu->nmi_pending = false;

cpu->cycles      = 0;     // Reset counters

cpu->instr_count = 0;

cpu->PC = READ16(VEC_RESET);  // Load PC from $FFFE–$FFFF

}

⚠  WARNING  cpu6802_reset() does NOT zero memory, does NOT reset SP, and does NOT clear A, B, X. Only PC, halt flag, interrupt flags and counters are affected. Memory contents from the previous session are preserved.

Sequence at 'r' command in the monitor:

1. cpu6802_reset(&cpu) is called

2. CC_I is set (IRQ masked)

3. halted, irq_pending, nmi_pending → false

4. cycles, instr_count → 0

5. PC = mem[$FFFE] << 8 | mem[$FFFF]

(If vectors not set → PC = $0000)

6. running = false  (set in sketch)

7. go_addr_set = false  (set in sketch)

# Chapter 7 — Serial Monitor Architecture

## 7.1  Input processing — non-blocking

The monitor reads characters one at a time from Serial inside loop(). It never blocks waiting for a complete line:

while (Serial.available()) {

char ch = (char)Serial.read();

if (ch == '\r') {

process_monitor_command(serial_buf);

serial_buf = ""; cr_received = true;

} else if (ch == '\n') {

if (cr_received) { cr_received = false; }  // ignore LF after CR

else { process_monitor_command(serial_buf); serial_buf = ""; }

} else {

cr_received = false;

Serial.print(ch);    // echo character back

if (serial_buf.length() < 80) serial_buf += ch;

}

}

ℹ  NOTE  serial_buf is limited to 80 characters. The buffer was increased from 32 (v6) to 80 (v7) to accommodate longer commands like 'save 0 0100 FFFF program_name'.

## 7.2  Three exclusive input modes

The monitor has three mutually exclusive modes. Only one is active at a time:

| Mode | Active when | How entered | How exited | Normal commands |
| --- | --- | --- | --- | --- |
| Normal | Default | Startup / after commands | N/A | Processed normally |
| Write mode | write_mode = true | w ADDR (no value) | Type '.' on a line | Ignored — bytes written to RAM |
| S19 mode | s19_mode = true | xsr command | '.' or S9 record received | Ignored — S19 records parsed |

## 7.3  process_monitor_command() — routing logic

process_monitor_command(cmd)

│

├─ Empty string?

│     write_mode: reprint $ADDR: prompt

│     s19_mode:   ignore

│     normal:     reprint '>'' prompt

│

├─ s19_mode == true?

│     '.' → s19_mode = false

│     'S' or 's' at start → s19_parse_line(cmd)

│       s19_parse_line returns true (S9 received) → s19_mode = false

│

├─ write_mode == true?

│     '.' → write_mode = false

│     else → parse hex byte, write to RAM at write_addr++

│

└─ Normal mode: first char routing

'r' → cpu6802_reset()

'g' → set PC, running=true

'p' → running=false, print state

's' → step N instructions

'd' → toggle debug_mode

'x' → cpu6802_print_state()

'v' → cmd_verify(addr, N)

'm' → cpu6802_dump_memory()

'w' → write mode or single byte

'i' → cpu6802_irq()

'n' → cpu6802_nmi()

'q' → print_help()

starts with 'slots' → cmd_slots()

starts with 'save'  → cmd_save()

starts with 'load'  → cmd_load()

starts with 'del'   → cmd_del()

starts with 'xs ' (not 'xsr') → cmd_s19_export()

starts with 'xsr'  → s19_mode = true

else → print unknown command error

## 7.4  cmd_verify() — disassembly display

The v command calls cmd_verify(start_addr, nb_lines). For each instruction it:

- Reads the opcode byte to determine instruction size from opcode_size[256]

- Prints the > marker if addr == cpu.PC

- Prints up to 3 opcode bytes, padded to fixed width

- Calls cpu6802_disasm() to get the mnemonic string

- Prints approximate cycle count as opcode_size + 1

static const uint8_t opcode_size[256] = {

1,1,1,1,1,1,1,1,1,1,1,1,1,1,1,1,  // $0x  (all 1-byte inherent)

1,1,1,1,1,1,1,1,1,1,1,1,1,1,1,1,  // $1x

2,1,2,2,2,2,2,2,2,2,2,2,2,2,2,2,  // $2x  ($20=BRA, 2 bytes)

// ...

3,3,3,3,3,3,3,3,3,3,3,3,3,3,3,3,  // $Bx  (all extended, 3 bytes)

};

# Chapter 8 — NVS Flash Storage System

## 8.1  NVS architecture on the ESP32

NVS (Non-Volatile Storage) is a key-value store implemented in the ESP32 Flash partition. It is accessed via the Arduino Preferences library. Data persists through power cuts and hardware resets.

ESP32 Flash memory layout (typical 4 MB chip):

Partition     Size    Contents

─────────     ─────   ─────────────────────────────────────────

nvs           24 KB   Key-value store (our slots go here)

phy_init       4 KB   RF calibration data

factory       1 MB    Arduino sketch (our compiled code)

spiffs / LFS  ~3 MB   (not used in this project)

## 8.2  NVS key naming scheme

The NVS namespace is '6802'. Each slot N (0–7) uses 4 keys:

| Key | Preferences method | Data type | Max size | Content |
| --- | --- | --- | --- | --- |
| s0_nm`–`s7_nm | putString / getString | String | 31 chars | Program name |
| s0_sa`–`s7_sa | putUShort / getUShort | uint16_t | 2 bytes | Start address |
| s0_ea`–`s7_ea | putUShort / getUShort | uint16_t | 2 bytes | End address (inclusive) |
| s0_dt`–`s7_dt | putBytes / getBytes | byte array | up to ~4 KB | Raw RAM bytes |

## 8.3  cmd_save() — write sequence

void cmd_save(uint8_t slot, uint16_t start, uint16_t end_addr, const char *name) {

uint16_t len = end_addr - start + 1;

// 1. Collect bytes from emulator RAM into a heap buffer

uint8_t *buf = (uint8_t *)malloc(len);

for (uint16_t i = 0; i < len; i++)

buf[i] = cpu6802_read(&cpu, start + i);

// 2. Write all 4 keys to NVS

prefs.begin("6802", false);   // false = read-write

prefs.putString(key_nm, name);

prefs.putUShort(key_sa, start);

prefs.putUShort(key_ea, end_addr);

prefs.putBytes(key_dt, buf, len);

prefs.end();

free(buf);

}

## 8.4  cmd_load() — read sequence

void cmd_load(uint8_t slot) {

prefs.begin("6802", true);  // true = read-only

if (!prefs.isKey(key_dt)) { prefs.end(); /* error */ return; }

uint16_t start    = prefs.getUShort(key_sa, 0x0100);

uint16_t end_addr = prefs.getUShort(key_ea, 0x01FF);

uint16_t len      = end_addr - start + 1;

uint8_t *buf = (uint8_t *)malloc(len);

prefs.getBytes(key_dt, buf, len);

prefs.end();

// Write bytes back to emulator RAM

for (uint16_t i = 0; i < len; i++)

cpu6802_write(&cpu, start + i, buf[i]);

free(buf);

go_addr = start;   // pre-set go_addr for immediate 'g'

go_addr_set = true;

}

⚠  WARNING  malloc() is used for the data buffer. On the ESP32 heap (~250 KB free after the sketch), this is safe for slot sizes up to several KB. For very large slots (> 10 KB), check heap availability with ESP.getFreeHeap() before calling.

## 8.5  Slot existence check

prefs.isKey(key_dt) checks whether the data key exists in NVS before attempting to read. This is the canonical way to detect an empty slot:

prefs.begin("6802", true);

bool occupied = prefs.isKey("s3_dt");  // true if slot 3 has data

prefs.end();

# Chapter 9 — S19 Motorola SREC Processor

## 9.1  S19 record anatomy

Complete S1 record breakdown:

S  1  1  3  0  1  0  0  8E 00 FF CE 02 00 ...16 bytes...  XX

│  │  └──┘  └──┴──┘  └────────────────────────────────┘  └─┘

│  │   BC   address         data bytes (BC-3 bytes)     checksum

│  └─ record type (1 = data with 16-bit address)

└─ 'S' start marker

BC (byte count) = 2 (address) + N (data bytes) + 1 (checksum)

For 16 data bytes: BC = 2+16+1 = 19 = 0x13

Checksum computation:

sum = BC + addr_hi + addr_lo + data[0] + ... + data[N-1]

checksum = (~sum) & 0xFF   (one's complement)

## 9.2  hexbyte() — hex string to byte

A local helper converts two hex ASCII characters to a byte, handling both upper and lower case:

static uint8_t hexbyte(const char *s, int pos) {

char hi = s[pos];

char lo = s[pos+1];

uint8_t h = (hi>='A') ? (hi-'A'+10) : (hi>='a') ? (hi-'a'+10) : (hi-'0');

uint8_t l = (lo>='A') ? (lo-'A'+10) : (lo>='a') ? (lo-'a'+10) : (lo-'0');

return (h << 4) | l;

}

## 9.3  s19_parse_line() — record processing

s19_parse_line(line)

│

├─ line.toUpperCase()

│

├─ type = s[1]

│

├─ type == '0' (S0 header)

│     Print acknowledgement, return false

│

├─ type == '1' (S1 data)

│     bc   = hexbyte(s, 2)

│     addr = hexbyte(s,4)<<8 | hexbyte(s,6)

│     datalen = bc - 3

│     ck_recv = hexbyte(s, 4+bc*2-2)

│     Verify checksum:

│       sum = bc + addr_hi + addr_lo + data[0..datalen-1]

│       ck_calc = (~sum) & 0xFF

│       ck_calc != ck_recv → print error, return false

│     Write datalen bytes to RAM starting at addr

│     Update s19_start/s19_end range trackers

│     return false

│

├─ type == '9' (S9 end)

│     go_addr = addr from record

│     go_addr_set = true

│     Print 'Import complete'

│     return true  ← signals 'exit S19 mode'

│

└─ other types (S2/S3/S5/S7/S8) → ignored, return false

## 9.4  cmd_s19_export() — record generation

Exports memory range [start, start+length-1] as a valid S19 file. Generates S0 (header), S1 records (16 bytes each), and S9 (end):

// S0 header: name bytes as data at address $0000

uint8_t bc0 = 2 + nlen + 1;  // 2=addr + name + 1=ck

uint8_t sum = bc0;            // addr=$0000 contributes 0

for each char in name: sum += char;

uint8_t ck0 = (~sum) & 0xFF;

// S1 data records: 16 bytes per record

bc = 2 + chunk + 1;

ck = s19_checksum(bc, addr, data, chunk);

Print: S1 + BC_hex + ADDR_hex + DATA_hex + CK_hex

// S9 end: start address as execution entry point

bc9 = 3;  // 2=addr + 1=ck

ck9 = s19_checksum(3, start, NULL, 0);

Print: S9 + 03 + START_hex + CK9_hex

# Chapter 10 — Built-in Disassembler

## 10.1  cpu6802_disasm() — design

The disassembler is a pure lookup function. It reads 3 bytes from memory (opcode, b1, b2) starting at addr, then decodes the opcode in a switch statement and formats the mnemonic into a caller-supplied buffer:

void cpu6802_disasm(CPU6802 *cpu, uint16_t addr, char *buf, int buf_len) {

uint8_t op = cpu6802_read(cpu, addr);

uint8_t b1 = cpu6802_read(cpu, addr+1);

uint8_t b2 = cpu6802_read(cpu, addr+2);

switch(op) {

case 0xA6: snprintf(buf, buf_len, "LDAA %d,X", b1); break;

case 0x27: snprintf(buf, buf_len, "BEQ $%04X", addr+2+(int8_t)b1); break;

// ...

default:   snprintf(buf, buf_len, "??? $%02X", op); break;

}

}

ℹ  NOTE  The disassembler DOES NOT advance PC. It reads speculatively. The 3-byte lookahead is safe because the 6802 never has an instruction longer than 3 bytes.

## 10.2  Branch target display

For branch instructions, the target address is pre-computed and displayed as an absolute address, making it much easier to read than raw offsets:

case 0x27: snprintf(buf, buf_len, "BEQ $%04X", addr+2+(int8_t)b1); break;

//                                              ├──────┤ ├────────┘

//                                              PC after  signed offset

//                                              the branch byte

## 10.3  Coverage — instruction groups implemented

| Group | Opcodes covered | Notes |
| --- | --- | --- |
| Inherent (1-byte) | NOP, TAP, TPA, INX, DEX, CLV, SEV, CLC, SEC, CLI, SEI, SBA, TAB, TBA, DAA, ABA, TSX, TXS, PSHA, PSHB, PULA, PULB, RTS, RTI, WAI, SWI, INCA, DECA, CLRA, INCB, DECB, CLRB, NEGB, TSTA, TSTB, ASLA, LSRA, ROLA, RORA, COMA, ASLB, LSRB, ROLB, RORB, COMB | All inherent instructions |
| LDAA | Imm $86, Dir $96, Idx $A6, Ext $B6 | All 4 modes |
| STAA | Dir $97, Idx $A7, Ext $B7 | 3 modes |
| LDAB | Imm $C6, Dir $D6, Idx $E6, Ext $F6 | All 4 modes |
| STAB | Dir $D7, Idx $E7, Ext $F7 | 3 modes |
| ADDA/ADDB/SUBA/SUBB | Imm/Dir/Idx/Ext for both | Full coverage |
| CMPA/CMPB/BITA/BITB | Imm/Dir/Idx/Ext | Full coverage |
| ANDA/ANDB/ORAA/ORAB/EORA/EORB | Imm/Dir/Idx/Ext | Full coverage |
| LDX/STX | Imm $CE, Dir $DE, Idx $EE, Ext $FE / Dir $9F, Idx $AF, Ext $BF | Full coverage |
| LDS/STS | Imm $8E, Dir $9E, Idx $AE, Ext $BE | Full coverage |
| CPX | Dir $9C, Idx $AC, Ext $BC, Imm $8C | Full coverage |
| JSR/BSR | Dir $9D, Idx $AD, Ext $BD, Rel $8D | All modes |
| INC/DEC mem | Idx $6C/$6A, Ext $7C/$7A | Memory versions |
| CLR mem | Idx $6F, Ext $7F | Memory versions |
| NEG mem | Idx $60, Ext $70 | Memory versions |
| TST mem | Idx $6D, Ext $7D | Memory versions |
| Shift/Rotate mem | LSR/ASR/ASL/ROL/ROR Idx/Ext | Full coverage |
| COM mem | Idx $63, Ext $73 | Memory versions |
| ADCA/ADCB/SBCA/SBCB | Imm/Dir/Idx/Ext | Full coverage |
| Branches | BRA, BEQ, BNE, BMI, BPL, BCC, BCS, BHI, BLS, BVC, BVS, BGE, BLT, BGT, BLE | All 15 branches |
| LDD/STD | Imm $CC, Dir $DC/$DD, Idx $EC/$ED, Ext $FC/$FD | Full coverage |
| SUBD/ADDD | Imm $83/$C3, Dir $93/$D3, Idx $A3/$E3, Ext $B3/$F3 | Full coverage |

# Chapter 11 — Troubleshooting Guide

## 11.1  Diagnostic tools available

| Tool | Command | What it reveals |
| --- | --- | --- |
| Register dump | x | A, B, X, SP, PC, CC, cycle count — check PC for runaway code |
| Disassembly | v PC | Instruction at current PC — check for ??? (unknown opcode) |
| Disassembly range | v ADDR N | Inspect code region — verify offsets and opcodes |
| Memory dump | m ADDR | 64 bytes raw hex — inspect stack, data, vectors |
| Step trace | s 1 | Execute one instruction with register state before and after |
| Debug mode | d | Every instruction printed — WARNING: slows emulator significantly |
| Interrupt vectors | m FFF8 | Raw bytes at $FFF8–$FFFF — verify all four vectors |
| Stack inspection | m SP | Dump stack area — count bytes pushed for subroutines vs interrupts |

## 11.2  Comprehensive problem table

| Symptom | Most likely cause | Root cause in code | Diagnosis | Fix |
| --- | --- | --- | --- | --- |
| CPU jumps to random address | Wrong branch offset | Branch offset is signed byte, calculated as target-(branch+2) | v ADDR — check BEQ/BNE/BRA offsets | Recalculate all offsets |
| Infinite loop / HELLO repeats forever | BEQ offset wrong — exit condition never met | off-by-one in offset calculation | v 0100 — BEQ should show correct target address | Recount: BEQ at $010A to $010F = offset 0x04, not 0x03 |
| Garbage on terminal after program | WAI used as halt instead of BRA $x | WAI pre-saves context but PC stays at WAI; loop continues to $0000 area | Remove WAI, add 20 FE (BRA itself) | Replace 3E with 20 FE |
| Terminal flood with null characters | Memory after code is all $00 which the UART loop sends | STAA $8000 in loop, no null-check or wrong null-check | v ADDR — look for missing CMPA #0 | Add CMPA #0 + BEQ exit |
| IRQ never fires | I flag is set (IRQ masked) | CPU starts with I=1 after init/reset | x — check CC, 'I' uppercase = masked | Add 0E (CLI) after LDS |
| IRQ fires once then never again | Handler sets I=1 but no CLI inside | do_interrupt() sets CC_I; handler must re-clear it for recurring IRQs | Check handler for 0E (CLI) | Add CLI at start of handler if recurring IRQs needed |
| Crash after JSR | Missing RTS | Stack grows without RTS popping return address | v subroutine_addr — check last instruction | Add 39 (RTS) as last byte of subroutine |
| All registers corrupted after interrupt | RTI missing — RTS used instead | RTS pops 2 bytes, RTI pops 7; stack offset becomes $0005 wrong | x — SP is 5 too high; wrong A,B,X,CC | Replace 39 with 3B (RTI) in handler |
| WAI hangs the system (no keyboard response) | Old bug — WAI without pc_before fix advanced PC | Fixed in current version — WAI loops on WAI | v — should show WAI at current PC | Re-upload if still on v6 |
| S19 checksum mismatch | Paste corruption (CR/LF added, spaces inserted) | hexbyte() reads wrong positions | Re-paste the failing S1 line exactly | Disable PuTTY 'Implicit CR' if needed |
| NMI triggers at boot | GPIO0 used as NMI pin | GPIO0 = BOOT button, pulled LOW at startup | Check PIN_NMI in .ino | Set PIN_NMI = -1 |
| ??? $xx in disassembly | Opcode not in disassembler switch | Disassembler is separate from executor — coverage may differ | Compare with executed behaviour — if CPU handles it, add to disasm | Add case 0xXX to cpu6802_disasm() switch |
| load corrupts program in RAM | Save range was wrong — missing code or data section | Bytes outside saved range are not in slot | slots — check range; m to verify after load | Re-save with correct start/end including all sections |
| w command types characters twice | Local echo ON in PuTTY | Characters echoed both by PuTTY and by the emulator's Serial.print(ch) | PuTTY → Connection → Terminal → Local echo = Force OFF | Set Force OFF |
| Unknown opcode $00 repeated | Code fell through into unwritten memory ($00 = ??? in emulator) | LDAA #$00 is valid but the default case handles unimplemented opcodes | Enable debug mode (d), check PC before $00 | Add halt or BRA loop at end of program |

## 11.3  Stack depth reference

Operation           Bytes pushed   Bytes popped   SP change

─────────────────   ────────────   ────────────   ──────────

JSR                 2 (PC)         —              SP -= 2

RTS                 —              2 (PC)         SP += 2

PSHA / PSHB         1 (A or B)     —              SP -= 1

PULA / PULB         —              1              SP += 1

do_interrupt (IRQ/NMI/SWI)  7 (PC,X,A,B,CC)  —   SP -= 7

RTI                 —              7              SP += 7

WAI (each call)     7              —              SP -= 7 EACH CALL

WARNING: WAI pushes context on every step() call while waiting.

With LDS #$00FF (SP=$00FF) and STEPS_PER_LOOP=200:

After 1 loop(): SP = $00FF - 7*200 = $00FF - $05B8 → wraps to $FA47

Stack overflow! Use BRA $x as halt instead of WAI.

## 11.4  Debug mode output format

When debug_mode = true (command d), every instruction prints:

PC:106 LDAA 0,X  A:0 B:0 X:200 SP:FF CC:11010011

│      │          │    │    │     │     └── CC in binary

│      │          │    │    │     └───── SP value

│      │          │    │    └─────────── X value

│      │          │    └──────────────── B value

│      │          └───────────────────── A value

│      └──────────────────────────────── disassembled mnemonic

└─────────────────────────────────────── PC BEFORE fetch (hex, no $)

ℹ  NOTE  All register values are printed in hex without leading zeros or 0x prefix, EXCEPT CC which is printed in binary by Serial.print(cpu->CC, BIN). A CC value of 0xD3 appears as '11010011'.

# Chapter 12 — Bug History and Version Changelog

## 12.1  Resolved bugs

| # | Version | Bug | Root cause | Fix applied |
| --- | --- | --- | --- | --- |
| 1 | v1 | Demo program saturated Serial — endless output | Program looped forever sending 'Hello 6802!\r\n' | Changed to print once then WAI |
| 2 | v2 | GPIO0 = ESP32 BOOT button caused immediate NMI loop at power-on | PIN_NMI was set to GPIO0; digitalRead(0)==LOW at boot | Set PIN_NMI = -1 (disabled) |
| 3 | v3 | Branch offsets incorrect — program jumped to wrong addresses | Offset formula misunderstood: target-(branch+2) not target-branch | Recalculated all demo program offsets in Python |
| 4 | v4 | WAI didn't freeze PC — stack overflow followed by random execution | WAI pushed context but pc_before not captured; PC advanced past WAI | Added pc_before = cpu->PC BEFORE FETCH8(); WAI restores cpu->PC = pc_before |
| 5 | v5 | CC flags displayed as garbage '%c%c%c%c%c%c' | Arduino Serial.print() does not support printf() format specifiers | Replaced with individual Serial.print(TST_FLAG(CC_x)?'X':'x') calls |
| 6 | v6 | Program B2 (HELLO) printed in infinite loop | BEQ offset was 0x03 instead of 0x04; loop exit skipped INX | Corrected offset to 0x04; replaced WAI with BRA $0112 (20 FE) |
| 7 | v6 | disassembler showed ??? $A6 for LDAA 0,X | Indexed mode opcodes not in disassembler switch | Added all 200+ opcodes/modes to cpu6802_disasm() |

## 12.2  Version history

| Version | Key changes |
| --- | --- |
| v1 | Initial implementation — basic CPU, hardcoded demo program |
| v2 | Removed GPIO0 NMI — PIN_NMI=-1. Demo prints once then stops. |
| v3 | All branch offsets corrected. Step command added. |
| v4 | WAI bug fixed (pc_before). Debug mode output corrected (individual Serial.print for CC flags). |
| v5 | Serial monitor hardened: local echo, line editing notes added to docs. |
| v6 | Clean emulator — no built-in program. v command with disasm. Opcode size table. |
| v7 (current) | NVS Flash slots (save/load/slots/del). S19 export (xs) and import (xsr). Disassembler fully expanded to 200+ opcode entries. Program B2 corrected. serial_buf increased to 80 chars. |

## 12.3  Known limitations

- WAI loops on itself — does not truly halt the bus. Stack overflows if program runs WAI and is never interrupted.

- Cycle counts are approximate — the step function assigns a fixed count per opcode, not the true variable counts of the real 6802.

- DAA (Decimal Adjust) is implemented but the V flag is left undefined after DAA, as per real 6802 behaviour.

- Unknown opcodes consume 2 cycles and continue execution instead of halting. Configurable by changing 'cycles=2' to 'cpu->halted=true'.

- The S19 import buffer length limit is 80 characters (serial_buf). S1 records with more than ~30 data bytes may be truncated. Standard S19 uses 16 bytes per record (56 characters) — within the limit.

- NVS slot size is limited by available NVS partition space (~20 KB total). Storing 8 slots of 2 KB each uses ~16 KB plus key overhead.

# Appendix A — Opcode Implementation Map

## A.1  Opcode grid — 6802 instruction set

Each cell shows: mnemonic (mode). Modes: # = immediate, D = direct, X = indexed, E = extended, R = relative, I = inherent. Shading: implemented in cpu6802.cpp.

|  | x0 | x1 | x2 | x3 | x4 | x5 | x6 | x7 | x8 | x9 | xA | xB | xC | xD | xE | xF |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0x | NOP I | — | — | — | — | — | TAP I | TPA I | INX I | DEX I | CLV I | SEV I | CLC I | SEC I | CLI I | SEI I |
| 1x | SBA I | — | — | — | — | — | TAB I | TBA I | — | DAA I | — | ABA I | — | — | — | — |
| 2x | BRA R | — | BHI R | BLS R | BCC R | BCS R | BNE R | BEQ R | BVC R | BVS R | BPL R | BMI R | BGE R | BLT R | BGT R | BLE R |
| 3x | TSX I | INS I | PULA I | PULB I | — | — | PSHA I | PSHB I | — | RTS I | ABX I | RTI I | — | — | WAI I | SWI I |
| 4x | NEGA I | — | — | COMA I | LSRA I | — | RORA I | ASRA I | ASLA I | ROLA I | DECA I | — | INCA I | TSTA I | — | CLRA I |
| 5x | NEGB I | — | — | COMB I | LSRB I | — | RORB I | ASRB I | ASLB I | ROLB I | DECB I | — | INCB I | TSTB I | — | CLRB I |
| 6x | NEG X | — | — | COM X | LSR X | — | ROR X | ASR X | ASL X | ROL X | DEC X | — | INC X | TST X | JMP X | CLR X |
| 7x | NEG E | — | — | COM E | LSR E | — | ROR E | ASR E | ASL E | ROL E | DEC E | — | INC E | TST E | JMP E | CLR E |
| 8x | SUBA # | CMPA # | SBCA # | SUBD # | ANDA # | BITA # | LDAA # | — | EORA # | ADCA # | ORAA # | ADDA # | CPX # | BSR R | LDS # | STS E |
| 9x | SUBA D | CMPA D | SBCA D | SUBD D | ANDA D | BITA D | LDAA D | STAA D | EORA D | ADCA D | ORAA D | ADDA D | CPX D | JSR D | LDS D | STX D |
| Ax | SUBA X | CMPA X | SBCA X | SUBD X | ANDA X | BITA X | LDAA X | STAA X | EORA X | ADCA X | ORAA X | ADDA X | CPX X | JSR X | LDS X | STX X |
| Bx | SUBA E | CMPA E | SBCA E | SUBD E | ANDA E | BITA E | LDAA E | STAA E | EORA E | ADCA E | ORAA E | ADDA E | CPX E | JSR E | LDS E | STX E |
| Cx | SUBB # | CMPB # | SBCB # | ADDD # | ANDB # | BITB # | LDAB # | — | EORB # | ADCB # | ORAB # | ADDB # | LDD # | — | LDX # | — |
| Dx | SUBB D | CMPB D | SBCB D | ADDD D | ANDB D | BITB D | LDAB D | STAB D | EORB D | ADCB D | ORAB D | ADDB D | LDD D | STD D | LDX D | STX D |
| Ex | SUBB X | CMPB X | SBCB X | ADDD X | ANDB X | BITB X | LDAB X | STAB X | EORB X | ADCB X | ORAB X | ADDB X | LDD X | STD X | LDX X | STX X |
| Fx | SUBB E | CMPB E | SBCB E | ADDD E | ANDB E | BITB E | LDAB E | STAB E | EORB E | ADCB E | ORAB E | ADDB E | LDD E | STD E | LDX E | STX E |

# Appendix B — Internal Macros Reference

| Macro | Defined in | Expansion / behaviour |
| --- | --- | --- |
| READ(addr) | cpu6802.cpp | cpu6802_read(cpu, addr) |
| WRITE(addr,v) | cpu6802.cpp | cpu6802_write(cpu, addr, (uint8_t)(v)) |
| READ16(addr) | cpu6802.cpp | cpu6802_read16(cpu, addr) |
| FETCH8() | cpu6802.cpp | READ(cpu->PC++) — read byte at PC, increment PC |
| FETCH16() | cpu6802.cpp | cpu->PC+=2, READ16(cpu->PC-2) — read 16-bit at PC, advance by 2 |
| SET_FLAG(f) | cpu6802.cpp | cpu->CC ¦= (f) |
| CLR_FLAG(f) | cpu6802.cpp | cpu->CC &= ~(f) |
| TST_FLAG(f) | cpu6802.cpp | ((cpu->CC & (f)) != 0) — returns bool |
| SET_OR_CLR(f,c) | cpu6802.cpp | if(c) SET_FLAG(f); else CLR_FLAG(f) |
| UPD_NZ8(val) | cpu6802.cpp | N=bit7(val&0xFF), Z=((val&0xFF)==0) |
| UPD_NZ16(val) | cpu6802.cpp | N=bit15(val&0xFFFF), Z=((val&0xFFFF)==0) |
| PUSH8(v) | cpu6802.cpp | WRITE(cpu->SP--, v&0xFF) — write then decrement |
| PULL8() | cpu6802.cpp | READ(++cpu->SP) — increment then read |
| PUSH16(v) | cpu6802.cpp | PUSH8(LSB) then PUSH8(MSB) — MSB at lower address |
| PULL16() | cpu6802.cpp | (PULL8()<<8) ¦ PULL8() — MSB first |
| CC_C | cpu6802.h | 0x01 — Carry bit mask |
| CC_V | cpu6802.h | 0x02 — Overflow bit mask |
| CC_Z | cpu6802.h | 0x04 — Zero bit mask |
| CC_N | cpu6802.h | 0x08 — Negative bit mask |
| CC_I | cpu6802.h | 0x10 — IRQ mask bit |
| CC_H | cpu6802.h | 0x20 — Half-carry bit mask |
| CC_BIT6 | cpu6802.h | 0x40 — Always-1 bit |
| CC_BIT7 | cpu6802.h | 0x80 — Always-1 bit |
| MEM_SIZE | cpu6802.h | 0x10000 — 64 KB total address space |
| RAM_INTERNAL | cpu6802.h | 128 — size of internal RAM array |
| IO_BASE | cpu6802.h | 0x8000 — start of I/O window |
| IO_END | cpu6802.h | 0x8FFF — end of I/O window |
| VEC_IRQ | cpu6802.h | 0xFFF8 |
| VEC_SWI | cpu6802.h | 0xFFFA |
| VEC_NMI | cpu6802.h | 0xFFFC |
| VEC_RESET | cpu6802.h | 0xFFFE |
| STEPS_PER_LOOP | esp32_6802.ino | 200 — instructions per loop() call |
| NVS_NS | esp32_6802.ino | "6802" — NVS namespace string |
| MAX_SLOTS | esp32_6802.ino | 8 — number of NVS save slots |

Copyright (c) 2026 Kamel Khelif  —  MIT License  —  Open-source software